Smart City Project - Setup Guide

This project simulates a smart city traffic management system using a Raspberry Pi, Flask for the web interface, and a 7-segment display for countdowns. It dynamically assigns traffic light timings based on random traffic conditions or sensor inputs and displays the data on a web dashboard. LEDs or GPIO-controlled devices simulate the traffic lights for four directions.

water management system uses a distance sensor (ultrasonic) to monitor water levels and a stepper motor to control water flow or operation based on these levels. The Flask web server provides an interface to view the current status and distance reading.

- Components and Tools Required

Hardware :

- RaspberryPi

- 7-Segment Display with 74HC595 Shift Register.

- Traffic Lights Simulation.

- Power Supply.

- Ultra Sonic Sensor.

- Stepper Motor.

The IoT kit provided by Xtrans Solution provides you the raspberry pi and Arduino models along with necessary sensors for the project.

Software :

- Python(3.10 or 3.11)

- Other python libraries(will be mentioned in installation.

- VS Code on laptop and RaspberryPi OS on raspberryPi.

- Set Up the Flask Application.

2. Libraries Used

- Flask: Used for creating the web application and serving HTML templates and JSON data.

- RPi.GPIO: Used for interacting with the GPIO pins on the Raspberry Pi to control hardware components like LEDs and 7-segment displays.

- threading: Used to run the traffic light simulation in a separate thread to keep the Flask web server responsive.

- time: Used for adding delays, timing the countdowns, and controlling the flow of the simulation.

- random: Used for generating random traffic light timings for simulation purposes.

Configuration Steps for Smart City Traffic  and Water Management System

  - 1.Install the Raspberry Pi OS.

  - Install Python and Required Libraries.

  - Connect the Hardware.

  - Run the Flask App.

    - python3 smart_city.py

  - Access the Web Interface.

http://<Raspberry_Pi_IP>:5000

- GPIO Pin Configuration: Double-check the GPIO pin numbers in your Python script against your wiring to ensure correctness.

- Test Hardware: Run simple scripts to test each component (e.g., stepper motor and sensor) before integrating them into the full project.

- Power Supply: Make sure the Raspberry Pi and all connected devices have a stable power source.

- Web Server Security: For more secure deployments, consider setting up password protection or using a reverse proxy.

These steps will help you configure and set up both projects successfully..

Snapshots

Fig 1 Running the traffic_manager1234.py

Fig 2. Web Page

Fig 3 Raspberry pi kit and 7segment lcd display

Fig 4.Running the water management pi code

Fig 5 Web Page

Fig 6 Motor is of when the distance is above 25cm

Fig 7 .Motor is on when the distance is above 10 cm and below 25 cm.